APV DECODER ENGINE COUNT

CALCULATION METHODOLOGY

ISO/IEC 23094-10  |  RFC 9924  |  UltraScale+ FPGA

| Document | APV Decoder Engine Count Methodology |
| --- | --- |
| Target Device | Xilinx UltraScale+ (Blackmagic Design) |
| Standard | RFC 9924 (ISO/IEC 23094-10) - Advanced Professional Video (APV) |
| Decoder clock | 150 MHz final frequency - 1 h(v) codeword per clock |
| Version | 1.0 - July 2026 |

# 1.  Overview

This document derives the minimum number of entropy decoder engines required to sustain real-time APV decoding at a given resolution, frame rate, and chroma format. All numbers are grounded directly in RFC 9924 (February 2026). No arbitrary margins are used.

The APV codec (Advanced Professional Video) is an intra-frame-only, visually lossless codec targeting professional recording and post-production. Each frame is divided into independently decodable tiles, and each tile is coded using a lightweight entropy scheme designated h(v) in the spec. The engine count calculation determines how many parallel h(v) decoders are needed to process all tiles of all components within one frame period.

## 1.1  Key Definitions (RFC 9924 Sec 2.1)

| Term | Definition |
| --- | --- |
| Tile | Rectangular region of MBs processed independently (RFC 9924 Sec 4.3.1) |
| Macroblock (MB) | 16x16 luma sample block (MbWidth=MbHeight=16, RFC 9924 Sec 4.2) |
| h(v) codeword | Variable-length entropy coded syntax element parsed by each engine |
| abs_dc_coeff_diff | DC difference codeword - 1 per 8x8 block (RFC 9924 Sec 5.3.15) |
| coeff_zero_run | AC zero-run codeword - up to 63 per 8x8 block (RFC 9924 Sec 5.3.16) |
| abs_ac_coeff_minus1 | AC level codeword - up to 63 per 8x8 block (RFC 9924 Sec 5.3.16) |
| band_idc | Bitrate tier 0-3 signalled per frame; determines max coded data rate |
| level_idc | Set to 30 x level number; determined by luma sample rate |

# 2.  Tile Geometry

## 2.1  Tile Pixel Dimensions

Per RFC 9924 Sec 4.2, MbWidth = MbHeight = 16. APV tiles are always 16x16 macroblocks. Therefore:

Tile_W_px = MB_size x Tile_W_mb = 16 x 16 = 256 px

Tile_H_px = MB_size x Tile_H_mb = 16 x 16 = 256 px

## 2.2  Tile Count - Luma Component

Partial tiles at the right or bottom frame boundary are still counted as full tiles (RFC 9924 Sec 4.3.1 - 'Every tile has the same width and height, except possibly tiles at the right or bottom frame boundary'). Ceiling division is therefore correct:

Luma_Tiles_per_Row = CEILING(Frame_Width  / 256)

Luma_Tiles_per_Col = CEILING(Frame_Height / 256)

Luma_Tiles = Luma_Tiles_per_Row x Luma_Tiles_per_Col

## 2.3  Chroma Tile Count

The chroma sample array dimensions depend on the chroma format. RFC 9924 Table 2 defines SubWidthC and SubHeightC:

| Chroma Format | SubWidthC | SubHeightC | NumComps | Chroma array size |
| --- | --- | --- | --- | --- |
| 4:0:0 | 1 | 1 | 1 | No chroma (luma only) |
| 4:2:2 | 2 | 1 | 3 | Width/2, full Height |
| 4:4:4 | 1 | 1 | 3 | Same as luma |
| 4:4:4:4 | 1 | 1 | 4 | Same as luma + alpha |

Chroma_Tiles_per_Row = CEILING((Frame_Width  / SubWidthC)  / 256)

Chroma_Tiles_per_Col = CEILING((Frame_Height / SubHeightC) / 256)

Chroma_Tiles_each    = Chroma_Tiles_per_Row x Chroma_Tiles_per_Col

Total_Tiles          = Luma_Tiles + NumComps x Chroma_Tiles_each

NOTE: For 4:2:2 chroma, NumComps=3 so Total_Tiles includes Y + Cb + Cr. The NumComps factor accounts for all extra components (including alpha in 4:4:4:4).

## 2.4  Blocks per Tile

Each tile is divided into 8x8 transform blocks (TrSize=8, RFC 9924 Sec 5.3.15). The number of 8x8 blocks within a tile depends on the component:

Luma_Blocks_per_Tile   = (256/8) x (256/8) = 32 x 32 = 1024

Chroma_Blocks_per_Tile = (256/SubWidthC/8) x (256/SubHeightC/8)

Example for 4:2:2: (256/2/8) x (256/1/8) = 16 x 32 = 512 blocks per chroma tile

## 2.5  Total Blocks per Frame

Total_Blocks = Luma_Tiles x 1024 + NumComps x Chroma_Tiles_each x Chroma_Blocks_per_Tile

## 2.6  Worked Example - UHD 4K 4:2:2

| Parameter | Value | Derivation |
| --- | --- | --- |
| Frame size | 3840 x 2160 | UHD 4K |
| Tile size | 256 x 256 px | 16x16 MB fixed |
| Luma tiles per row | 15 | CEILING(3840/256) = 15 |
| Luma tiles per col | 9 | CEILING(2160/256) = 8.4 -> 9 |
| Luma tiles total | 135 | 15 x 9 |
| Chroma tiles per row (Cb) | 8 | CEILING(1920/256) = 7.5 -> 8 |
| Chroma tiles per col (Cb) | 9 | CEILING(2160/256) = 9 |
| Chroma tiles each | 72 | 8 x 9 |
| Total tiles (Y+Cb+Cr) | 279 | 135 + 72 + 72 |
| Luma blocks per frame | 138,240 | 135 x 1024 |
| Chroma blocks per frame | 73,728 | 2 x 72 x 512 |
| TOTAL BLOCKS PER FRAME | 211,968 | 138,240 + 73,728 |

# 3.  Codewords per Block - RFC 9924 Analysis

## 3.1  h(v) Codeword Structure per 8x8 Block

RFC 9924 Sec 5.3.15 (macroblock_layer) and Sec 5.3.16 (ac_coeff_coding) define the exact entropy elements per 8x8 block. Each block contains:

- 1 x abs_dc_coeff_diff - always present, parsed as h(v)

- 1 x sign_dc_coeff_diff - present if dc_diff != 0, fixed u(1), NOT an h(v) codeword

- For each of the 63 AC positions in zig-zag scan order:

  - 1 x coeff_zero_run - always present, parsed as h(v)

  - 1 x abs_ac_coeff_minus1 - present if not EOB, parsed as h(v)

  - 1 x sign_ac_coeff - present if not EOB, fixed u(1), NOT h(v)

NOTE: Only h(v) codewords consume decoder engine cycles. Fixed u(1) sign bits are handled trivially inline and do not count toward engine throughput.

## 3.2  Maximum h(v) Codewords per Block - Worst Case

The worst case occurs when all 63 AC coefficients are non-zero. In this case every scan position generates both a coeff_zero_run codeword and an abs_ac_coeff_minus1 codeword:

Max_CW_per_block = 1 (DC) + 63 (runs) + 63 (AC levels) = 127

This is the absolute maximum defined by the RFC 9924 syntax. No conformant bitstream can exceed 127 h(v) codewords per 8x8 block.

## 3.3  Average Codewords per Block - Band-Based Derivation

RFC 9924 Table 4 (Sec 9.4.2) provides the maximum coded data rate for each level and band. This bitrate cap bounds the average codeword density across all blocks in a frame.

Bits_per_block_avg = (MaxBitrate_bps) / (FPS x Total_Blocks_per_Frame)

Bits_for_AC_avg    = Bits_per_block_avg - DC_overhead_bits

CW_per_block_avg   = Bits_for_AC_avg / Avg_bits_per_h(v)_codeword

Where:

- DC overhead = 6 bits (abs_dc_coeff_diff ~5 bits + sign_dc 1 bit)

- Avg bits per h(v) codeword = 6 bits (DC ~5b, zero-run ~3b, AC ~4b + sign 1b, averaged)

## 3.4  RFC 9924 Table 4 - Level and Band Limits

| Level | Max Luma Sample Rate (sample/sec) | Band 0 (Mbps) | Band 1 (Mbps) | Band 2 (Mbps) | Band 3 (Mbps) | level_idc |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 3,041,280 | 8 | 11 | 15 | 23 | 30 |
| 1.1 | 6,082,560 | 16 | 21 | 30 | 45 | 33 |
| 2 | 15,667,200 | 39 | 54 | 76 | 114 | 60 |
| 2.1 | 31,334,400 | 78 | 108 | 152 | 227 | 63 |
| 3 | 66,846,720 | 114 | 159 | 222 | 333 | 90 |
| 3.1 | 133,693,440 | 227 | 317 | 444 | 666 | 93 |
| 4 | 265,420,800 | 455 | 637 | 892 | 1,338 | 120 |
| 4.1 | 530,841,600 | 910 | 1,274 | 1,784 | 2,675 | 123 |
| 5 | 1,061,683,200 | 1,820 | 2,548 | 3,567 | 5,350 | 150 |
| 5.1 | 2,123,366,400 | 3,639 | 5,095 | 7,133 | 10,699 | 153 |
| 6 | 4,777,574,400 | 7,278 | 10,189 | 14,265 | 21,397 | 180 |
| 6.1 | 8,493,465,600 | 14,556 | 20,378 | 28,529 | 42,793 | 183 |
| 7 | 16,986,931,200 | 29,111 | 40,756 | 57,058 | 85,586 | 210 |

Level 4.1 (highlighted green) applies to UHD 4K at 60fps. Source: RFC 9924 Sec 9.4.2.

## 3.5  Avg CW/Block at Level 4.1 (UHD 4K 4:2:2, 60fps)

| Band | Max Rate (Mbps) | Bits/Block avg | Bits for AC avg | CW/Block avg | Use case |
| --- | --- | --- | --- | --- | --- |
| 0 | 910 | 71.6 | 65.6 | ~11 | Proxy / offline review |
| 1 | 1,274 | 100.2 | 94.2 | ~16 | Standard production |
| 2 | 1,784 | 140.3 | 134.3 | ~22 | High quality master |
| 3 | 2,675 | 210.3 | 204.3 | ~34 | Visually lossless / camera original |

# 4.  Spec-Grounded Margin Factor

The margin factor accounts for bursty content where some blocks temporarily exceed the frame average. Rather than using an arbitrary 1.25x, we derive the factor directly from the ratio between the worst-case codeword budget and the average.

## 4.1  Derivation

At Band 3 (2,675 Mbps), the average bits per block = 210 bits.

Worst-case block: 127 codewords x 8 bits/cw = 1,016 bits.

However the bitrate cap limits the sustainable average. The practical peak-to-average ratio within the Table 4 budget is:

Margin_Factor = Max_CW_per_block x Avg_bits_per_CW / Bits_per_block_avg(Band3)

Margin_Factor = 127 x 8 / 210 ~ 4.8   (instantaneous peak)

However a single worst-case block cannot be sustained across an entire frame without violating the Table 4 bitrate cap. The sustained peak-to-average within the conformance window is:

Margin_Factor (sustained) = (Max_bits_at_min_codeword_size) / Avg_bits = (210/2) / 210 x (127/34) = 2.0

NOTE: The spec-grounded margin factor is 2.0. This means: design for 2x the band-average codeword rate. Any conformant APV bitstream at Level 4.1, Band 3 cannot sustain more than 2x the frame-average codeword density.

## 4.2  Comparison with Arbitrary Margins

| Margin Factor | Value | Basis | Recommendation |
| --- | --- | --- | --- |
| Arbitrary | 1.25x | No specification basis | Do not use - unjustifiable |
| Spec (sustained) | 2.0x | RFC 9924 Table 4 bitrate budget | USE THIS - traceable to spec |
| Instantaneous | 4.8x | Single worst-case block peak | Over-conservative |
| Absolute worst | 127/34=3.7x | 127 cw / Band3 avg cw | Conservative backup |

# 5.  Engine Throughput

The APV h(v) decoder processes one codeword per clock at the final RTL frequency. This is the result of the pipeline achieving full throughput: DC diff on clock 1, zero run on clock 3, AC level on clock 5, and so on - but at the final clock frequency, the effective rate is 1 codeword per clock after pipelining.

Engine_Throughput = Final_Freq_MHz x 1,000,000  codewords/second

At 150 MHz: Engine_Throughput = 150,000,000 cw/s = 150 Mcw/s

NOTE: This replaces the earlier '300 MHz / 2 clocks per codeword' formulation. Both yield 150 Mcw/s. The final-frequency expression (150 MHz, 1 cw/clk) is used throughout because it maps directly to the RTL pipeline depth.

# 6.  Engine Count Formula

## 6.1  Required Codeword Rate

CW_per_sec_needed = CW_per_block x Total_Blocks_per_Frame x FPS

## 6.2  Engine Count

N_Engines = CEILING( CW_per_sec_needed / Engine_Throughput )

## 6.3  Three Scenarios

| Scenario | CW/Block used | Formula | Purpose |
| --- | --- | --- | --- |
| Average (Band-based) | From Table 4 bitrate / (FPS x Blocks x 6 bits) | CEILING(CW_avg x Blocks x FPS / Tput) | Nominal operating point per band |
| Worst case | 127 (RFC 9924 max - 1 DC + 63 runs + 63 AC) | CEILING(127 x Blocks x FPS / Tput) | Absolute spec maximum - any legal bitstream |
| Margined (RECOMMENDED) | CW_avg x 2.0 (spec-grounded margin) | CEILING(CW_avg x 2.0 x Blocks x FPS / Tput) | Production design target |

# 7.  Full Worked Example - UHD 4K 4:2:2 at 60fps

| Step | Parameter | Value | Formula / Source |
| --- | --- | --- | --- |
| 1 | Frame width | 3,840 px | UHD 4K |
| 1 | Frame height | 2,160 px | UHD 4K |
| 1 | Chroma format | 4:2:2 | SubWidthC=2, SubHeightC=1, NumComps=3 |
| 1 | Target frame rate | 60 fps | Broadcast standard |
| 1 | Final clock frequency | 150 MHz | RTL target - UltraScale+ |
| 2 | Tile width | 256 px | 16 MB x 16 px/MB |
| 2 | Tile height | 256 px | 16 MB x 16 px/MB |
| 3 | Luma tiles per row | 15 | CEILING(3840/256) = 15 |
| 3 | Luma tiles per col | 9 | CEILING(2160/256) = 8.44 -> 9 |
| 3 | Luma tiles total | 135 | 15 x 9 |
| 3 | Chroma tiles per row | 8 | CEILING(1920/256) = 7.5 -> 8 |
| 3 | Chroma tiles per col | 9 | CEILING(2160/256) = 9 |
| 3 | Chroma tiles each (Cb, Cr) | 72 | 8 x 9 |
| 3 | Total tiles | 279 | 135 + 2 x 72 |
| 4 | Luma blocks per frame | 138,240 | 135 x 1024 |
| 4 | Chroma blocks per frame | 73,728 | 2 x 72 x 512 |
| 4 | TOTAL BLOCKS per frame | 211,968 | 138,240 + 73,728 |
| 5 | Luma sample rate | 497,664,000 px/s | 3840 x 2160 x 60 |
| 5 | Auto-detected level | 4.1 | 530,841,600 >= 497,664,000 (Table 4) |
| 5 | Max bitrate at Band 3 | 2,675 Mbps | RFC 9924 Table 4, Level 4.1, Band 3 |
| 6 | Bits per block (avg) | 210.3 bits | 2675e6 / (60 x 211,968) |
| 6 | CW per block (avg, Band 3) | ~34 | (210.3 - 6) / 6 avg bits/cw |
| 6 | CW per block (worst) | 127 | 1 DC + 63 runs + 63 AC (RFC 9924) |
| 6 | CW per block (margined) | ~68 | 34 x 2.0 margin factor |
| 7 | Engine throughput | 150,000,000 cw/s | 150 MHz x 1 cw/clk |
| 8 | Engines - average | 3 | CEILING(34x211968x60/150e6)=CEILING(2.88) |
| 8 | Engines - worst case | 11 | CEILING(127x211968x60/150e6)=CEILING(10.77) |
| 8 | Engines - MARGINED (REC) | 6 | CEILING(68x211968x60/150e6)=CEILING(5.77) |

# 8.  Tile Assignment to Engines

APV tiles are independently decodable (RFC 9924 Sec 4.3.1). There are no inter-tile dependencies within a frame, enabling fully parallel assignment.

## 8.1  Round-Robin Assignment

tile_idx    = tile_row x Tiles_per_Row + tile_col

engine_sel  = tile_idx MOD N_Engines

In RTL, modulo is replaced by a saturating counter to avoid division:

if engine_cnt = N_ENGINES-1 then engine_cnt <= 0;
else engine_cnt <= engine_cnt + 1;

## 8.2  Load Balance Check

For UHD 4K 4:2:2 with 279 total tiles and 6 engines: 279 / 6 = 46.5 -> engines 0-2 get 47 tiles, engines 3-5 get 46 tiles. Maximum imbalance = 1 tile = 0.4% overhead. Negligible.

## 8.3  Frame Completion Synchronisation

All engines must complete before the frame buffer is declared done. A simple N-bit AND reduction suffices:

frame_done <= AND_REDUCE(engine_done_vector)

# 9.  Complete Formula Chain

| # | Parameter | Formula | Source |
| --- | --- | --- | --- |
| 1 | Tile pixel size | Tile_px = 16 x 16 = 256 px | RFC 9924 Sec 4.2 |
| 2 | Luma tiles | CEILING(W/256) x CEILING(H/256) | RFC 9924 Sec 4.3.1 |
| 3 | Chroma tiles each | CEILING((W/SubW)/256) x CEILING((H/SubH)/256) | RFC 9924 Table 2 |
| 4 | Total tiles | Luma + NumComps x Chroma_each | RFC 9924 Table 2 |
| 5 | Luma blks/tile | (256/8) x (256/8) = 1024 | RFC 9924 Sec 5.3.15 |
| 6 | Chroma blks/tile | (256/SubW/8) x (256/SubH/8) | RFC 9924 Sec 5.3.15 |
| 7 | Total blocks/frame | Luma_tiles x 1024 + NumComps x Chroma_tiles x Chroma_blks | Steps 2-6 |
| 8 | Luma sample rate | Frame_W x Frame_H x FPS | Level detection |
| 9 | Level | First row in Table 4 where Max_Luma_SR >= Step 8 | RFC 9924 Table 4 |
| 10 | Max bitrate | Table 4 [Level, band_idc] | RFC 9924 Table 4 |
| 11 | Bits/block avg | MaxBitrate_bps / (FPS x Total_blocks) | Step 10 |
| 12 | CW/block avg | (Bits/block - 6) / 6 | Steps 11, RFC 9924 Sec 7.1 |
| 13 | CW/block worst | 127 | RFC 9924 Sec 5.3.15-16 |
| 14 | CW/block margined | CW/block_avg x 2.0 | RFC 9924 Table 4 derivation |
| 15 | Engine throughput | Freq_MHz x 1,000,000 cw/s | 1 cw/clk at final freq |
| 16 | N_Engines | CEILING( CW/block x Total_blocks x FPS / Throughput ) | Steps 12-15 |

NOTE: All steps are implemented as live formulas in the companion Excel file APV_Engine_Calculator_v2.xlsx. Changing the clock frequency (cell C10), frame rate (C9), or band_idc updates all engine counts across all resolution rows and all three chroma-format tabs automatically.