APV DECODER ENGINE COUNT

CALCULATION METHODOLOGY  (v2 - Corrected)

ISO/IEC 23094-10  |  RFC 9924  |  UltraScale+ FPGA

| Document | APV Decoder Engine Count Methodology v2 (Corrected) |
| --- | --- |
| Standard | RFC 9924 (ISO/IEC 23094-10) - Advanced Professional Video (APV) |
| Target Device | Xilinx UltraScale+ FPGA |
| Decoder clock | 150 MHz final frequency - 1 h(v) codeword per engine per clock |
| Status | Corrected - block count uses MB-first formula per RFC 9924 Sec 4.2 |
| Version | 2.0 - July 2026 |

CORRECTIONS vs v1:  (1) Block count now uses MB-first formula per RFC 9924 Sec 4.2: UHD 4K 4:2:2 = 259,200 blocks (was 211,968).  (2) Worst-case engines = 14 (was 11).  (3) 2.0x margin factor relabelled as engineering assumption - NOT RFC-grounded.

# 1.  Overview

This document derives the minimum number of entropy decoder engines required to sustain real-time APV decoding at a given resolution, frame rate, and chroma format. All block-count formulas are grounded in RFC 9924 Sec 4.2. Engineering assumptions are clearly labelled as such.

The APV codec uses an intra-frame-only structure. Each frame is partitioned into tiles, each tile coded with the h(v) variable-length entropy scheme. Engine count is determined by the total h(v) codeword rate that must be sustained per second.

## 1.1  Key Definitions (RFC 9924 Sec 2.1 and 4.2)

| Term | Definition |
| --- | --- |
| Macroblock (MB) | 16x16 luma sample block: MbWidth=MbHeight=16 (RFC 9924 Sec 4.2) |
| MbWidthC | Chroma MB width = 16/SubWidthC (RFC 9924 Sec 4.2) |
| MbHeightC | Chroma MB height = 16/SubHeightC (RFC 9924 Sec 4.2) |
| Tile | Frame partition containing Y + chroma tile_data for ALL components (RFC 9924 Sec 4.3.1) |
| h(v) codeword | Variable-length entropy coded element: abs_dc_coeff_diff, coeff_zero_run, abs_ac_coeff_minus1 |
| band_idc | Bitrate tier 0-3 signalled per frame - determines max coded data rate (RFC 9924 Sec 9.4) |
| level_idc | 30 x level number - determined by luma sample rate (RFC 9924 Sec 9.4) |

# 2.  Block Count - Corrected MB-First Method (RFC 9924 Sec 4.2)

CORRECTION: v1 of this document counted blocks using a tile-first approach that under-counted chroma blocks. The correct method derives blocks directly from the macroblock count, which is the normative basis in RFC 9924 Sec 4.2.

## 2.1  Chroma Subsampling Parameters (RFC 9924 Table 2)

| Format | SubWidthC | SubHeightC | NumComps | MbWidthC | MbHeightC |
| --- | --- | --- | --- | --- | --- |
| 4:2:2 | 2 | 1 | 3 | 8 | 16 |
| 4:4:4 | 1 | 1 | 3 | 16 | 16 |
| 4:4:4:4 | 1 | 1 | 4 | 16 | 16 |

## 2.2  Blocks per Macroblock

Each macroblock contributes the following 8x8 blocks (RFC 9924 Sec 4.2, MbWidthC=16/SubWidthC, MbHeightC=16/SubHeightC):

Y blocks/MB  = (MbWidth/8)  x (MbHeight/8)  = (16/8) x (16/8) = 4  (all formats)

Cb blocks/MB = (MbWidthC/8) x (MbHeightC/8) = (16/SubWidthC/8) x (16/SubHeightC/8)

Cr blocks/MB = same as Cb

A  blocks/MB = 4  (4:4:4:4 only - alpha component same as luma)

| Format | Y blks/MB | Cb blks/MB | Cr blks/MB | A blks/MB | TOTAL blks/MB |
| --- | --- | --- | --- | --- | --- |
| 4:2:2 | 4 | 2 | 2 | 0 | 8 |
| 4:4:4 | 4 | 4 | 4 | 0 | 12 |
| 4:4:4:4 | 4 | 4 | 4 | 4 | 16 |

## 2.3  Total Blocks per Frame

MB_cols       = CEILING(Frame_Width  / 16)

MB_rows       = CEILING(Frame_Height / 16)

MBs_per_frame = MB_cols x MB_rows

Total_blocks  = MBs_per_frame x Blocks_per_MB

## 2.4  Corrected Block Counts - All Resolutions

| Resolution | Width | Height | MB cols | MB rows | Blks/frame 4:2:2 | Blks/frame 4:4:4 | Blks/frame 4:4:4:4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 720p | 1280 | 720 | 80 | 45 | 28,800 | 43,200 | 57,600 |
| FHD 1080 | 1920 | 1080 | 120 | 68 | 65,280 | 97,920 | 130,560 |
| FHD 2K | 2048 | 1080 | 128 | 68 | 69,632 | 104,448 | 139,264 |
| UHD 4K | 3840 | 2160 | 240 | 135 | 259,200 | 388,800 | 518,400 |
| DCI 4K | 4096 | 2160 | 256 | 135 | 276,480 | 414,720 | 552,960 |
| UHD 8K | 7680 | 4320 | 480 | 270 | 1,036,800 | 1,555,200 | 2,073,600 |
| 8K Full | 8192 | 4320 | 512 | 270 | 1,105,920 | 1,658,880 | 2,211,840 |

UHD 4K 4:2:2 corrected from 211,968 (v1) to 259,200 (v2). All rows use MB-first formula.

# 3.  h(v) Codewords per Block (RFC 9924 Sec 5.3.15-16)

Each 8x8 block in macroblock_layer() contains the following h(v) coded elements:

- 1 x abs_dc_coeff_diff: always present, coded as h(v)

- 1 x sign_dc_coeff_diff: if dc != 0, fixed u(1) - NOT h(v), does not consume engine cycles

- For each of 63 AC scan positions:

  - 1 x coeff_zero_run: always present, coded as h(v)

  - 1 x abs_ac_coeff_minus1: present if not EOB, coded as h(v)

  - 1 x sign_ac_coeff: present if not EOB, fixed u(1) - NOT h(v)

NOTE: Only h(v) elements count toward engine cycles. Fixed-width u(1) sign bits are handled inline.

Max h(v) CW/block = 1 (DC) + 63 (zero-runs) + 63 (AC levels) = 127

This 127 is the hard upper bound. No conformant APV bitstream can exceed it per block. (RFC 9924 Sec 5.3.15-16)

# 4.  RFC 9924 Table 4 - Level and Band Limits

| Level | Max Luma Sample Rate (s/s) | Band 0 (Mbps) | Band 1 (Mbps) | Band 2 (Mbps) | Band 3 (Mbps) | level_idc | 4K60? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3,041,280 | 8 | 11 | 15 | 23 | 30 |  |
| 1.1 | 6,082,560 | 16 | 21 | 30 | 45 | 33 |  |
| 2 | 15,667,200 | 39 | 54 | 76 | 114 | 60 |  |
| 2.1 | 31,334,400 | 78 | 108 | 152 | 227 | 63 |  |
| 3 | 66,846,720 | 114 | 159 | 222 | 333 | 90 |  |
| 3.1 | 133,693,440 | 227 | 317 | 444 | 666 | 93 |  |
| 4 | 265,420,800 | 455 | 637 | 892 | 1,338 | 120 |  |
| 4.1 | 530,841,600 | 910 | 1,274 | 1,784 | 2,675 | 123 | YES |
| 5 | 1,061,683,200 | 1,820 | 2,548 | 3,567 | 5,350 | 150 |  |
| 5.1 | 2,123,366,400 | 3,639 | 5,095 | 7,133 | 10,699 | 153 |  |
| 6 | 4,777,574,400 | 7,278 | 10,189 | 14,265 | 21,397 | 180 |  |
| 6.1 | 8,493,465,600 | 14,556 | 20,378 | 28,529 | 42,793 | 183 |  |
| 7 | 16,986,931,200 | 29,111 | 40,756 | 57,058 | 85,586 | 210 |  |

Level 4.1 (green) applies to UHD 4K 60fps: 497,664,000 luma samples/s < 530,841,600 limit.

# 5.  Bitrate-Derived Average CW/Block

NOTE: The following derivation uses engineering assumptions (avg bits/cw = 6, DC overhead = 6 bits). These are NOT specified by RFC 9924 and must be validated against actual RTL measurements.

Bits_per_block_avg = MaxBitrate_bps / (FPS x Total_Blocks)

CW_per_block_avg   = (Bits_per_block_avg - DC_overhead_bits) / avg_bits_per_cw

## 5.1  UHD 4K 4:2:2, Level 4.1, 60fps - Corrected

| Band | Max Rate (Mbps) | Bits/Block avg | Bits for AC avg | CW/Block avg | Use case |
| --- | --- | --- | --- | --- | --- |
| 0 | 910 | 58.5 | 52.5 | ~9 | Proxy / offline |
| 1 | 1,274 | 81.9 | 75.9 | ~13 | Standard production |
| 2 | 1,784 | 114.7 | 108.7 | ~18 | High quality master |
| 3 | 2,675 | 172.0 | 166.0 | ~28 | Visually lossless (corrected from ~34) |

# 6.  Engineering Margin Factor

CORRECTION: The 2.0x margin used in v1 was claimed to be RFC-grounded. This is incorrect. RFC 9924 constrains the coded data rate but does NOT define a maximum ratio between local codeword density and frame-average density. The 2.0x factor is an engineering assumption.

The margin is retained as a practical engineering target but must be labelled accordingly:

CW_per_block_margined = CW_per_block_avg x Engineering_Margin_Factor

The factor of 2.0 means: the design handles bursts up to 2x the frame average codeword density. This must be validated with measured and adversarial content - it is not guaranteed by the spec.

| Margin Factor | Value | Basis | Status |
| --- | --- | --- | --- |
| Arbitrary | 1.25x | No basis at all | Do not use |
| Engineering target | 2.0x | Practical design choice | USE - but label as engineering assumption |
| Instantaneous peak | 4.8x | Single worst-case block | Over-conservative |
| Spec worst case | 127/28 = 4.5x | 127 cw / Band3 avg cw | RFC-grounded but hardware impractical |

# 7.  Engine Throughput

Engine_Throughput = Final_Freq_MHz x 1,000,000  codewords/second

At 150 MHz: Engine_Throughput = 150,000,000 cw/s = 150 Mcw/s

NOTE: Implementation assumption: the RTL pipeline sustains 1 h(v) codeword per clock including bit extraction, state update, and sign-bit handling. Verify for long VLCs, word-boundary crossings, and backpressure.

# 8.  Engine Count Formula and Results

## 8.1  Formula

N_Engines = CEILING( CW_per_block x Total_Blocks_per_Frame x FPS / Engine_Throughput )

## 8.2  Full Results - UHD 4K 4:2:2, Level 4.1, Band 3, 60fps, 150MHz

| Scenario | CW/block | CW/sec needed | Engines | Basis |
| --- | --- | --- | --- | --- |
| Nominal average (Band 3) | ~28 | ~436M | 3 | Engineering estimate |
| Engineering margined (2x) | ~56 | ~872M | 6 | Engineering target - needs validation |
| Absolute worst case | 127 | 1,975M | 14 | RFC 9924 hard upper bound |

Design decision: 6 engines is a valid engineering production target if supported by burst buffering and adversarial bitstream testing. 14 engines is the RFC-guaranteed worst case. 6 is NOT an RFC-proven upper bound.

# 9.  Engine Counts - All Resolutions (Corrected)

## 9.1  4:2:2 (8 blocks/MB)

| Resolution | Blks/frame | CW/blk avg Band 3 | CW/sec avg | Engines avg | Engines marg (x2) | Engines worst(127) |
| --- | --- | --- | --- | --- | --- | --- |
| 720p | 28,800 | ~9 | 15.6M | 1 | 1 | 2 |
| FHD 1080 | 65,280 | ~14 | 54.7M | 1 | 1 | 4 |
| FHD 2K | 69,632 | ~14 | 58.3M | 1 | 1 | 4 |
| UHD 4K | 259,200 | ~28 | 435.7M | 3 | 6 | 14 |
| DCI 4K | 276,480 | ~26 | 430.2M | 3 | 6 | 15 |
| UHD 8K | 1,036,800 | ~28 | 1,742.9M | 12 | 23 | 53 |
| 8K Full | 1,105,920 | ~26 | 1,720.7M | 12 | 23 | 57 |

# 10.  Complete Corrected Formula Chain

| # | Parameter | Formula | Source |
| --- | --- | --- | --- |
| 1 | MB cols | CEILING(Frame_Width / 16) | RFC 9924 Sec 4.2 |
| 2 | MB rows | CEILING(Frame_Height / 16) | RFC 9924 Sec 4.2 |
| 3 | MBs/frame | MB_cols x MB_rows | RFC 9924 Sec 4.2 |
| 4 | Blks/MB | 4(Y) + 2x(16/SubW/8)x(16/SubH/8) + (nc-3)x4 | RFC 9924 Sec 4.2 Table 2 |
| 5 | Total blks | MBs/frame x Blks/MB | Steps 3-4 |
| 6 | Luma s/s | Frame_W x Frame_H x FPS | Level detection |
| 7 | Level | First Table 4 row where Max_Luma_SR >= step 6 | RFC 9924 Table 4 |
| 8 | Max bitrate | Table 4 [Level, band_idc] | RFC 9924 Table 4 |
| 9 | Bits/blk | MaxBitrate_bps / (FPS x Total_blks) | Step 8 (eng. assump.) |
| 10 | CW/blk avg | (Bits/blk - 6) / 6 | Engineering assumption |
| 11 | CW/blk max | 127 | RFC 9924 Sec 5.3.15-16 |
| 12 | CW/blk marg | CW/blk_avg x 2.0 | Engineering assumption |
| 13 | Throughput | Freq_MHz x 1,000,000 cw/s | 1 cw/clk at final freq |
| 14 | N_Engines | CEILING(CW/blk x Total_blks x FPS / Throughput) | Steps 10-13 |

NOTE: Steps 9-10 and 12 are engineering assumptions. Steps 1-8, 11, 13 are directly traceable to RFC 9924.