APV Bitstream
Header Fields Reference

ISO/IEC 23094-10  ·  RFC 9924  ·  UltraScale+ VHDL Implementation

Each header structure is a separate NAL unit; bit offsets are relative to the first byte of that NAL unit (i.e. byte 0x00 = start-code byte 0).  The notation 0xBB.b means byte BB, starting at bit b within that byte (MSB = bit 0).  Fields marked ★ are of particular interest for the 10-bit UltraScale+ decoder target.

| Field Name | Syntax Element | Bits | Bit Offset | Value / Range | Description |
| --- | --- | --- | --- | --- | --- |
| Access Unit Delimiter (AUD) |  |  |  |  |  |
| AUD Start Code | aud_start_code | 32 | 0x00 | 0x00000001 | MPEG-style start code prefix marking the start of the AUD NAL unit |
| AUD NAL Type | aud_nal_unit_type | 8 | 0x04 | 0x00 | Identifies this as an Access Unit Delimiter unit |
| Primary Pic Type | primary_pic_type | 3 | 0x05 | 0–7 | Signals the slice types present in the AU (e.g. 0 = I-slices only) |
| Reserved | reserved_zero_5bits | 5 | 0x05.3 | 0x00 | Must be zero; reserved for future use |
| Frame Parameter Set (FPS) |  |  |  |  |  |
| FPS Start Code | fps_start_code | 32 | 0x00 | 0x00000001 | Start code prefix for the FPS NAL unit |
| FPS NAL Type | fps_nal_unit_type | 8 | 0x04 | 0x01 | Identifies this as a Frame Parameter Set unit |
| FPS ID | frame_parameter_set_id | 6 | 0x05 | 0–63 | Identifies the FPS; referenced by all slice headers in the AU |
| Profile IDC | profile_idc | 8 | 0x05.6 | 0–255 | Specifies the APV coding profile (e.g. 0 = 422-10) |
| Level IDC | level_idc | 8 | 0x06.6 | 0–255 | Specifies the APV level, bounding processing and buffer demands |
| Band IDC | band_idc | 4 | 0x07.6 | 0–15 | Specifies the APV band (bitrate tier within the level) |
| ★ Bit Depth Minus 8 | bit_depth_minus8 | 4 | 0x08.2 | 0, 2 (→ 8/10-bit) | Luma & chroma bit depth = value + 8. 0 → 8-bit; 2 → 10-bit (design target) |
| Chroma Format IDC | chroma_format_idc | 4 | 0x08.6 | 1=4:2:0 / 2=4:2:2 / 3=4:4:4 | Specifies the chroma subsampling format |
| Capture Time Distance | capture_time_distance | 8 | 0x09.2 | 0–255 | Nominal frame-to-frame capture time in units of 1/fps |
| Colour Primaries | colour_primaries | 8 | 0x0A.2 | ITU-T H.273 | Specifies colour primaries per H.273 (e.g. 9 = BT.2020) |
| Transfer Characteristics | transfer_characteristics | 8 | 0x0B.2 | ITU-T H.273 | Transfer function (e.g. 16 = ST 2084 / PQ; 18 = HLG) |
| Matrix Coefficients | matrix_coefficients | 8 | 0x0C.2 | ITU-T H.273 | Luma/chroma derivation matrix (e.g. 9 = BT.2020 non-const) |
| Video Full Range | video_full_range_flag | 1 | 0x0D.2 | 0 / 1 | 0 = studio swing (limited); 1 = full range |
| FPS Extension Flag | fps_extension_flag | 1 | 0x0D.3 | 0 / 1 | If 1, an fps_extension() structure follows immediately |
| Frame Header (FH) |  |  |  |  |  |
| FH Start Code | fh_start_code | 32 | 0x00 | 0x00000001 | Start code prefix for the Frame Header NAL unit |
| FH NAL Type | fh_nal_unit_type | 8 | 0x04 | 0x02 | Identifies this as a Frame Header unit |
| FPS ID Ref | frame_parameter_set_id | 6 | 0x05 | 0–63 | References the active FPS for this frame |
| Spatial Reference | spatial_ref_info | 8 | 0x05.6 | var | Encodes spatial reference information for the tile layout |
| Frame Width Minus1 | frame_width_minus1 | 16 | 0x06.6 | 0–65535 | Luma frame width in samples, minus 1 |
| Frame Height Minus1 | frame_height_minus1 | 16 | 0x08.6 | 0–65535 | Luma frame height in samples, minus 1 |
| Tile Width Minus1 | tile_width_minus1 | 12 | 0x0A.6 | 0–4095 | Tile width in luma samples minus 1 (must be aligned to 16 pels) |
| Tile Height Minus1 | tile_height_minus1 | 12 | 0x0C.2 | 0–4095 | Tile height in luma samples minus 1 (must be aligned to 16 pels) |
| Tile Header (TH) — per tile, offset relative to start of each TH |  |  |  |  |  |
| Tile Header Size | tile_header_size | 16 | 0x00 | 0–65535 | Size in bytes of this tile header structure |
| Tile Index | tile_idx | 16 | 0x02 | 0–N−1 | Raster-scan index of this tile within the frame |
| QP (Y) | qp_y | 8 | 0x04 | 0–51 | Quantisation parameter for the luma (Y) component |
| QP (Cb) | qp_cb | 8 | 0x05 | 0–51 | QP for the Cb chroma component |
| QP (Cr) | qp_cr | 8 | 0x06 | 0–51 | QP for the Cr chroma component |
| QP (Alpha) | qp_alpha | 8 | 0x07 | 0–51 | QP for the alpha channel (present when chroma_format_idc signals alpha) |
| Tile Data Size (Y) | tile_data_size_y | 32 | 0x08 | bytes | Byte length of the sealed luma entropy-coded data block |
| Tile Data Size (Cb) | tile_data_size_cb | 32 | 0x0C | bytes | Byte length of the sealed Cb entropy-coded data block |
| Tile Data Size (Cr) | tile_data_size_cr | 32 | 0x10 | bytes | Byte length of the sealed Cr entropy-coded data block |
| Tile Data Size (A) | tile_data_size_alpha | 32 | 0x14 | bytes | Byte length of the alpha data block (when present) |

Notes:  (1) bit_depth_minus8 sits at 0x08.2 within the FPS NAL unit, spanning bits 66–69.  For the 10-bit design target, its value is 2.  (2) Sub-byte bit offsets arise because the 6-bit fps_id field at 0x05.0 is not byte-aligned.  (3) Tile Header offsets reset to 0x00.0 for each individual tile.  (4) All reserved fields must be zero on write and silently ignored on read per RFC 9924.